1.Plan du réseau : Cisco

IP Machines Clientes et VLAN :

VLAN 10 -> 192.168.10.0			VLAN 20 -> 192.168.20.0

g0/0.1 : 192.168.10.1			g0/0.2 : 192.168.20.1

PC0 : 192.168.10.2				PC1 : 192.168.20.2

WEB : 192.168.10.3

VLAN 30 -> 192.168.30.0			VLAN 40 -> 192.168.40.0

g0/0.3 : 192.168.30.1			g0/0.4 :192.168.40.1

PC2 : 192.168.30.2				PC3 : 192.168.40.2

2. Configuration VLAN, DNS, DHCP (cisco)

Switch :           - Création des VLAN

-    Attribution des noms et des numéros des VLAN en fonction de leurs interfaces

vlan 10

name ADMIN

vlan 20

name PERSONNEL						### Création des VLAN

vlan 30

name PRODUCTION

vlan 40

name VIDEO

*************************************

interface FastEthernet 0/1

switchport access Vlan 10

switchport mode access

interface FastEthernet 0/2

switchport access Vlan 10

switchport mode access

interface FastEthernet 0/3

switchport access Vlan 20

switchport mode access				### Association d’int

aux VLAN

interface FastEthernet 0/4

switchport access Vlan 30

switchport mode access

interface FastEthernet 0/5

switchport access Vlan 40

switchport mode access

interface FastEthernet 0/6

switchport mode trunk

Routeur : 	    -   Configuration en mode trunk, et  DHCP, DNS

ip dhcp pool VLAN10

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

ip dhcp pool VLAN20

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

ip dhcp pool VLAN30

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

ip dhcp pool VLAN40

network 192.168.40.0 255.255.255.0

default-router 192.168.40.1

*************************************

interface GigabitEthernet0/0

no ip address

duplex auto

speed auto

interface GigabitEthernet0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

interface GigabitEthernet0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

interface GigabitEthernet0/0.30

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

interface GigabitEthernet0/0.40

encapsulation dot1Q 40

ip address 192.168.40.1 255.255.255.0

3. Ping, Capture de trames (Cisco)

Nous avons rajouté des machines clientes pour ces tests.

Routeur vers client :

CLI du Routeur

Client vers Routeur :

CLI d’une machine cliente (PC0 : 192.168.10.2) présente dans le VLAN 10

Client vers Client :

CLI d’une machine cliente (PC2 : 192.168.30.2) présente dans le vlan 30