Building configuration...

Current configuration : 1696 bytes

!

version 15.1

no service timestamps log datetime msec

no service timestamps debug datetime msec

no service password-encryption

!

hostname Router

!

!

!

!

!

ip dhcp pool vlan10

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 10.0.0.1

ip dhcp pool vlan20

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

ip dhcp pool vlan30

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

ip dhcp pool vlan40

network 192.168.40.0 255.255.255.0

default-router 192.168.40.1

!

!

!

ip cef

no ipv6 cef

!

!

!

!

license udi pid CISCO2911/K9 sn FTX1524R1WS-

!

!

!

!

!

!

!

!

!

!

!

spanning-tree mode pvst

!

!

!

!

!

!

interface GigabitEthernet0/0

no ip address

duplex auto

speed auto

!

interface GigabitEthernet0/0.1

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

ip access-group 10 in

!

interface GigabitEthernet0/0.2

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

ip access-group 20 in

!

interface GigabitEthernet0/0.3

encapsulation dot1Q 30

ip address 192.168.30.1 255.255.255.0

ip access-group 30 in

!

interface GigabitEthernet0/0.4

encapsulation dot1Q 40

ip address 192.168.40.1 255.255.255.0

ip access-group 40 in

!

interface GigabitEthernet0/1

no ip address

duplex auto

speed auto

shutdown

!

interface GigabitEthernet0/2

no ip address

duplex auto

speed auto

shutdown

!

interface Vlan1

no ip address

shutdown

!

ip classless

!

ip flow-export version 9

!

!

access-list 10 permit 192.168.10.0 0.0.0.255

access-list 20 permit 192.168.20.0 0.0.0.255

access-list 30 permit 192.168.30.0 0.0.0.255

access-list 40 permit 192.168.40.0 0.0.0.255

!

!

!

!

!

line con 0

!

line aux 0

!

line vty 0 4

login

!

!

!

end