Building configuration...

Current configuration : 4105 bytes

!

! Last configuration change at 13:51:37 UTC Wed Jun 19 2024

!

version 15.8

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname Paris

!

boot-start-marker

boot-end-marker

!

!

!

no aaa new-model

!

!

!

!

!

!

!

!

ip dhcp excluded-address 192.168.20.5

ip dhcp excluded-address 192.168.20.6

ip dhcp excluded-address 192.168.20.12

ip dhcp excluded-address 192.168.20.15

!

ip dhcp pool vlan10

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 192.168.20.5

!

ip dhcp pool vlan20

network 192.168.20.0 255.255.255.0

default-router 192.168.20.1

dns-server 192.168.20.5

!

ip dhcp pool vlan30

network 192.168.30.0 255.255.255.0

default-router 192.168.30.1

dns-server 192.168.20.5

!

!

!

ip domain name societe5.fr

ip cef

no ipv6 cef

multilink bundle-name authenticated

!

!

!

license udi pid C927-4P sn FGL2348L2DL

!

!

username admin privilege 15 secret 5 $1$QA6v$s8/bNhasPej/5A..kNZvV1

!

redundancy

!

!

controller VDSL 0

!

!

class-map match-any DATA

match ip dscp default

class-map match-any VOICE

match ip dscp ef

class-map match-any SIGNALING

match protocol sip

!

policy-map VOICE_POLICY

class VOICE

priority percent 50

class SIGNALING

bandwidth percent 10

class DATA

bandwidth percent 40

!

!

!

!

!

interface ATM0

no ip address

shutdown

no atm ilmi-keepalive

!

interface Ethernet0

no ip address

shutdown

!

interface GigabitEthernet0

switchport access vlan 30

switchport mode access

no ip address

shutdown

!

interface GigabitEthernet1

switchport trunk allowed vlan 1-1005

switchport mode trunk

no ip address

!

interface GigabitEthernet2

switchport access vlan 20

switchport mode access

no ip address

shutdown

!

interface GigabitEthernet3

description VLAN10 Interface

switchport access vlan 10

switchport mode access

no ip address

shutdown

!

interface GigabitEthernet4

ip address 10.5.1.1 255.240.0.0

ip nat outside

ip virtual-reassembly in

duplex auto

speed auto

!

interface Vlan1

no ip address

shutdown

!

interface Vlan3

no ip address

shutdown

!

interface Vlan10

ip address 192.168.10.1 255.255.255.0

ip access-group VLAN10_TO_VLAN30 in

ip nat inside

ip virtual-reassembly in

!

interface Vlan20

ip address 192.168.20.1 255.255.255.0

ip access-group VLAN20_TO_OTHERS in

ip nat inside

ip virtual-reassembly in

!

interface Vlan30

ip address 192.168.30.1 255.255.255.0

ip access-group VLAN30_TO_VLAN10 in

ip nat inside

ip virtual-reassembly in

!

ip forward-protocol nd

no ip http server

no ip http secure-server

!

!

ip nat inside source list NAT_VLAN10 interface GigabitEthernet4 overload

ip nat inside source list NAT_VLAN20 interface GigabitEthernet4 overload

ip nat inside source list NAT_VLAN30 interface GigabitEthernet4 overload

ip nat inside source static tcp 192.168.20.6 80 interface GigabitEthernet4 80

ip route 0.0.0.0 0.0.0.0 10.0.0.1

ip ssh version 2

!

ip access-list extended NAT_VLAN10

permit ip 192.168.10.0 0.0.0.255 any

ip access-list extended NAT_VLAN20

permit ip 192.168.20.0 0.0.0.255 any

ip access-list extended NAT_VLAN30

permit ip 192.168.30.0 0.0.0.255 any

ip access-list extended VLAN20_TO_VLAN30

deny   icmp 192.168.20.0 0.0.0.255 192.168.30.0 0.0.0.255

permit ip any host 192.168.10.5

permit ip any host 192.168.10.8

permit ip any host 192.168.10.9

permit ip any host 192.168.10.10

permit ip any host 192.168.40.2

deny   icmp 192.168.20.0 0.0.0.255 192.168.10.0 0.0.0.255

permit ip any any

ip access-list extended VLAN30_TO_VLAN20

deny   icmp 192.168.30.0 0.0.0.255 192.168.20.0 0.0.0.255

permit ip any host 192.168.10.5

permit ip any host 192.168.10.8

permit ip any host 192.168.10.9

permit ip any host 192.168.10.10

permit ip any host 192.168.40.2

deny   icmp 192.168.30.0 0.0.0.255 192.168.10.0 0.0.0.255

permit ip any any

!

!

!

tftp-server flash:/firmware/vadsl_module_img.bin

!

control-plane

!

!

line con 0

line 4

no activation-character

transport preferred none

transport input all

transport output all

stopbits 1

line vty 0 4

login local

transport input ssh

transport output ssh

!

scheduler allocate 20000 1000

!

end